Question 4. Exercise 1: TCP Echo Server and Client

Server (tcp_echo_server.c)

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <unistd.h>

#include <arpa/inet.h>

#define PORT 8080

#define BUF_SIZE 1024

int main() {

int server_fd, new_socket;

struct sockaddr_in address;

char buffer[BUF_SIZE] = {0};

server_fd = socket(AF_INET, SOCK_STREAM, 0);

address.sin_family = AF_INET;

address.sin_addr.s_addr = INADDR_ANY;

address.sin_port = htons(PORT);

bind(server_fd, (struct sockaddr *)&address, sizeof(address));

listen(server_fd, 3);

int addrlen = sizeof(address);

new_socket = accept(server_fd, (struct sockaddr *)&address, (socklen_t*)&addrlen);

read(new_socket, buffer, BUF_SIZE);

printf("Received: %s\n", buffer);

send(new_socket, buffer, strlen(buffer), 0);

close(new_socket);

close(server_fd);

return 0;

}

Client (tcp_echo_client.c)

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <unistd.h>

#include <arpa/inet.h>

#define PORT 8080

int main() {

int sock = 0;

struct sockaddr_in serv_addr;

char *msg = "Hello Server!";

char buffer[1024] = {0};

sock = socket(AF_INET, SOCK_STREAM, 0);

serv_addr.sin_family = AF_INET;

serv_addr.sin_port = htons(PORT);

inet_pton(AF_INET, "127.0.0.1", &serv_addr.sin_addr);

connect(sock, (struct sockaddr *)&serv_addr, sizeof(serv_addr));

send(sock, msg, strlen(msg), 0);

read(sock, buffer, 1024);

printf("Echo from server: %s\n", buffer);

close(sock);

return 0;

}

Question 5 : Exercises using NS2 Network Simulator

Exercise 1: Setting Up of Various Network TopologiesTCL Script (Star Topology – Wired)

# Create simulator

set ns [new Simulator]

# Open files

set tracefile [open out.tr w]

$ns trace-all $tracefile

# Create nodes

set n0 [$ns node]

set n1 [$ns node]

set n2 [$ns node]

set n3 [$ns node]

# Create links

$ns duplex-link $n0 $n1 1Mb 10ms DropTail

$ns duplex-link $n0 $n2 1Mb 10ms DropTail

$ns duplex-link $n0 $n3 1Mb 10ms DropTail

# Run simulation

$ns at 1.0 "finish"

proc finish {} {

global ns tracefile

$ns flush-trace

close $tracefile

exit 0

}

$ns run

Exercise 2: Implementation of Various MAC Protocols

set val(chan) Channel/WirelessChannel

set val(prop) Propagation/TwoRayGround

set val(mac) Mac/802_11

set val(ifq) Queue/DropTail/PriQueue

set val(ll) LL

set val(netif) Phy/WirelessPhy

set val(nn) 2

# Simulator and topology

set ns [new Simulator]

set topo [new Topography]

$topo load_flatgrid 500 500

# Create nodes

for {set i 0} {$i < $val(nn)} {incr i} {

set node_($i) [$ns node]

$node_($i) random-motion 0

}

# Link nodes and assign MAC protocol

$ns at 1.0 "$node_(0) setdest 250 250 10"

$ns at 2.0 "$node_(1) setdest 260 260 10"

Exercise 3: Measurement of Routing Protocols

How to Specify Routing Protocol in Wireless

set val(rp) AODV ;# You can change to DSDV or DSR

Exercise 4: Analysis of TCP/IP Protocol under Various Mechanisms

Sample TCP Setup

set tcp [new Agent/TCP/Reno]

set sink [new Agent/TCPSink]

$ns attach-agent $n0 $tcp

$ns attach-agent $n1 $sink

$ns connect $tcp $sink

Exercise 5: Setup a Network with Application Protocols and Analyze Performance

Application (FTP over TCP)

set ftp [new Application/FTP]

$ftp attach-agent $tcp

$ns at 1.0 "$ftp start"

$ns at 5.0 "$ftp stop"

for CBR over UDP:

set udp [new Agent/UDP]

set null [new Agent/Null]

$ns attach-agent $n0 $udp

$ns attach-agent $n1 $null

$ns connect $udp $null

set cbr [new Application/Traffic/CBR]

$cbr set packetSize_ 1000

$cbr set rate_ 1Mb

$cbr attach-agent $udp

$ns at 1.0 "$cbr start"

Question 6: Comparison of TCP/IP, Socket, Pipes. Analyse which is the best

## TCP/IP Communication using Sockets (C)

### TCP Server (tcp_server.c)

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <unistd.h>

#include <netinet/in.h>

#define PORT 8080

int main() {

int server_fd, new_socket;

struct sockaddr_in address;

int addrlen = sizeof(address);

char buffer[1024] = {0};

server_fd = socket(AF_INET, SOCK_STREAM, 0);

address.sin_family = AF_INET;

address.sin_addr.s_addr = INADDR_ANY;

address.sin_port = htons(PORT);

bind(server_fd, (struct sockaddr*)&address, sizeof(address));

listen(server_fd, 3);

printf("Server is waiting for connection...\n");

new_socket = accept(server_fd, (struct sockaddr*)&address, (socklen_t*)&addrlen);

read(new_socket, buffer, 1024);

printf("Received from client: %s\n", buffer);

send(new_socket, buffer, strlen(buffer), 0);

close(new_socket);

close(server_fd);

return 0;

}

TCP Client (tcp_client.c)

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <unistd.h>

#include <arpa/inet.h>

#define PORT 8080

int main() {

int sock = 0;

struct sockaddr_in serv_addr;

char *msg = "Hello from client!";

char buffer[1024] = {0};

sock = socket(AF_INET, SOCK_STREAM, 0);

serv_addr.sin_family = AF_INET;

serv_addr.sin_port = htons(PORT);

inet_pton(AF_INET, "127.0.0.1", &serv_addr.sin_addr);

connect(sock, (struct sockaddr*)&serv_addr, sizeof(serv_addr));

send(sock, msg, strlen(msg), 0);

read(sock, buffer, 1024);

printf("Echo from server: %s\n", buffer);

close(sock);

return 0;

}

- Pipes (Anonymous IPC between Parent and Child)

- pipe_example.c

- #include <stdio.h>

- #include <unistd.h>

- #include <string.h>

- #include <sys/wait.h>

- int main() {

- int pipefd[2];

- pid_t pid;

- char write_msg[] = "Message from parent";

- char read_msg[100];

- pipe(pipefd); // create pipe

- pid = fork();

- if (pid == 0) {

- // Child process

- close(pipefd[1]); // close write end

- read(pipefd[0], read_msg, sizeof(read_msg));

- printf("Child received: %s\n", read_msg);

- close(pipefd[0]);

- } else {

- // Parent process

- close(pipefd[0]); // close read end

- write(pipefd[1], write_msg, strlen(write_msg)+1);

- wait(NULL); // wait for child

- close(pipefd[1]);

- }

- return 0;

- }